import random

import matplotlib.pyplot as plt

import math

from scipy.stats import norm

import numpy as np

from scipy.signal import fftconvolve

import itertools

def get_symbols(b):

b = np.asarray(b)

val = 1 / np.sqrt(2)

pairs = b.reshape(-1, 2)

symbols = np.empty(len(pairs), dtype=complex)

# create masks for each pair

mask00 = (pairs[:,0] == 0) & (pairs[:,1] == 0)

mask01 = (pairs[:,0] == 0) & (pairs[:,1] == 1)

mask10 = (pairs[:,0] == 1) & (pairs[:,1] == 0)

mask11 = (pairs[:,0] == 1) & (pairs[:,1] == 1)

symbols[mask00] = complex(+val, +val)

symbols[mask01] = complex(-val, +val)

symbols[mask10] = complex(+val, -val)

symbols[mask11] = complex(-val, -val)

return symbols

def get_noise(pn, n, sps):

value = 1/np.sqrt(2)

total = sps*n//2

re = np.random.normal(0, 1, total)

im = np.random.normal(0, 1, total)

noise = (re * value * np.sqrt(pn)) + 1j * (im * value * np.sqrt(pn))

return noise

def detector(ds):

ds = np.asarray(ds, dtype=complex)

val = 1/math.sqrt(2)

detected = np.zeros_like(ds, dtype=complex)

mask = (ds.real > 0) & (ds.imag > 0)

detected[mask] = complex(val, val)

mask = (ds.real < 0) & (ds.imag >= 0)

detected[mask] = complex(-val, val)

mask = (ds.real < 0) & (ds.imag < 0)

detected[mask] = complex(-val, -val)

mask = (ds.real >= 0) & (ds.imag < 0)

detected[mask] = complex(val, -val)

return detected

def get_received_bits(symbols):

symbols = np.asarray(symbols, dtype=complex)

N = len(symbols)

# Preallocate array for 2 bits per symbol

r_bits = np.zeros(2 * N, dtype=int)

# First quadrant: real > 0, imag > 0 → bits 00

mask = (symbols.real > 0) & (symbols.imag > 0)

r_bits[2*mask.nonzero()[0]] = 0  # first bit

r_bits[2*mask.nonzero()[0]+1] = 0  # second bit

mask = (symbols.real < 0) & (symbols.imag >= 0)

r_bits[2*mask.nonzero()[0]] = 0

r_bits[2*mask.nonzero()[0]+1] = 1

mask = (symbols.real < 0) & (symbols.imag < 0)

r_bits[2*mask.nonzero()[0]] = 1

r_bits[2*mask.nonzero()[0]+1] = 1

mask = (symbols.real >= 0) & (symbols.imag < 0)

r_bits[2*mask.nonzero()[0]] = 1

r_bits[2*mask.nonzero()[0]+1] = 0

return r_bits

def get_power(s):

s = np.asarray(s)

return np.mean(np.abs(s)**2)

def show_y(y):

# t = np.arange(len(y))

plt.figure(figsize=(10,6))

plt.subplot(2,1,1)

plt.plot(y.real)

plt.title('Real Part')

plt.grid()

plt.subplot(2,1,2)

plt.plot(y.imag)

plt.title('Imaginary Part')

plt.grid()

plt.tight_layout()

plt.show()

def eye_diagram(signal, sps):

L = 3 * sps

plt.figure(figsize=(10,6))

plt.subplot(2,1,1)

for i in range(0, len(signal) - L, sps):

segment = signal[i:i+L]

plt.plot(segment.real, color='blue', alpha=0.3)

plt.title("Eye Diagram - Real (I) part")

plt.xlabel("Samples")

plt.ylabel("Amplitude")

plt.grid(True)

plt.subplot(2,1,2)

for i in range(0, len(signal) - L, sps):

segment = signal[i:i+L]

plt.plot(segment.imag, color='red', alpha=0.3)

plt.title("Eye Diagram - Imag (Q) part")

plt.xlabel("Samples")

plt.ylabel("Amplitude")

plt.grid(True)

plt.tight_layout()

plt.show()

def constellation(ds):

plt.figure(figsize=(6,6))

plt.scatter(ds.real, ds.imag, color='blue', s=50, alpha=0.7)

# Draw axes

plt.axhline(0, color='black', linewidth=1)

plt.axvline(0, color='black', linewidth=1)

plt.title("Constellation Diagram (QPSK)")

plt.xlabel("In-Phase (I)")

plt.ylabel("Quadrature (Q)")

plt.grid(True)

plt.axis('equal')

plt.show()

def fix_h(sps, n):

h = np.zeros(sps*n//2)

h[:sps] = 1 / sps

return h

def show_pulses(pulses):

fig, (ax_real, ax_imag) = plt.subplots(2, 1, figsize=(10, 5), sharex=True)

ax_real.plot(pulses.real, 'o', color='blue', label='Real Part')

ax_real.set_ylabel('Real')

ax_real.grid(True)

ax_real.legend()

ax_imag.plot(pulses.imag, 'o', color='red', label='Imag Part')

ax_imag.set_ylabel('Imag')

ax_imag.set_xlabel('Sample Index')

ax_imag.grid(True)

ax_imag.legend()

plt.tight_layout()

plt.show()

def question_2(h, sps, n):

snr = 10**(0/10)

noise = get_noise(1/snr, n, sps)

y = fftconvolve(noise, h, mode='full')[:sps*n//2]

y_samples = y[sps-1::sps]

print(f"Noise Power: {np.mean(np.abs(y_samples)**2)}")

def question_3(pulses, h, sps, n):

num = 10

snr = 10**(0/10)

noise = get_noise(1/snr, n, sps)

s_n = pulses + noise

y = fftconvolve(pulses, h, mode='full')[:sps*n//2]

y_noise = fftconvolve(s_n, h, mode='full')[:sps*n//2]

m1 = np.abs(y[:num])

m2 = np.abs(y_noise[:num])

plt.figure(figsize=(10,5))

plt.plot(np.arange(num), m1, 'b-o', label='Καθαρό σήμα (|y|)')

plt.plot(np.arange(num), m2, 'r-o', label='Σήμα με θόρυβο (|y|)')

plt.title("Μέγεθος ληφθέντος σήματος για τα πρώτα 10 σύμβολα")

plt.xlabel("Δείγματα")

plt.ylabel("Μέγεθος |y|")

plt.grid(True)

plt.legend()

plt.show()

def question_4(pulses, h, sps, n, bits):

snr = 10**(0/10)

noise = get_noise(1/snr, n, sps)

s_n = pulses + noise

y = fftconvolve(s_n, h, mode='full')[:sps*n//2]

ds = y[sps-1::sps]

detected = detector(ds)

received_bits = get_received_bits(detected)

bit_errors = bits != received_bits

num = np.sum(bit_errors)

print(f"There where {num} errors")

def question_5(pulses, h, sps, n):

snr = 10**(12.5/10)

noise = get_noise(1/snr, n, sps)

s_n = pulses + noise

y = fftconvolve(s_n, h, mode='full')[:sps*n//2]

ds = y[sps-1::sps]

ds = ds[:100]

constellation(ds)

def question_6(h, sps):

all_bits = list(itertools.product([0,1], repeat=6))

plt.figure(figsize=(10,6))

for bits in all_bits:

bits = np.array(bits)

symbols = get_symbols(bits)

pulses = np.repeat(symbols, sps)

y = fftconvolve(pulses, h, mode='full')[:len(pulses)]

L = 3*sps

plt.plot(y.real[:L], color='blue', alpha=0.3)

plt.plot(y.imag[:L], color='red', alpha=0.3)

plt.title("Eye Diagram για όλα τα δυνατά bit 0/1 (3 σύμβολα)")

plt.xlabel("Δείγματα")

plt.ylabel("Πλάτος")

plt.grid(True)

plt.show()

def question_7(pulses, h, sps, n):

snr = 10**(12.5/10)

noise = get_noise(1/snr, n, sps)

s_n = pulses + noise

y = fftconvolve(s_n, h, mode='full')[:sps*n//2]

y = y[:100]

eye_diagram(y, sps)

def question_8(symbols, pulses, h, sps, n, snr_db):

pse = np.zeros(len(snr_db))

ser = np.zeros(len(snr_db))

for idx, i in enumerate(snr_db):

snr = 10**(i/10)

noise = get_noise(1/snr, n, sps)

s_n = pulses + noise

y = fftconvolve(s_n, h, mode='full')[:sps*n//2]

ds = y[sps-1::sps]

detected = detector(ds)

symbol_errors = symbols != detected

num = np.sum(symbol_errors)

# pse.append(num/n)

pse[idx] = num / len(symbols)

x = np.sqrt(snr)

q = norm.sf(x)

ser[idx] = 2*q - q**2

plt.figure(figsize=(8,5))

plt.semilogy(snr_db, ser, '-', label="Theoretical SER")

pse = np.maximum(pse, 1e-6)

plt.semilogy(snr_db, pse, '*', label="Simulated SER")

plt.xlabel("SNR (dB)")

plt.ylabel("Probability of Error")

plt.title("SER vs SNR")

plt.grid(True, which='both', linestyle='--', alpha=0.6)

plt.legend()

plt.show()

def question_9(bits, pulses, h, sps, n, snr_db):

pbe = np.zeros(len(snr_db))

ber = np.zeros(len(snr_db))

for idx, i in enumerate(snr_db):

snr = 10**(i/10)

noise = get_noise(1/snr, n, sps)

s_n = pulses + noise

y = fftconvolve(s_n, h, mode='full')[:sps*n//2]

ds = y[sps-1::sps]

detected = detector(ds)

received_bits = get_received_bits(detected)

bit_errors = bits != received_bits

num = np.sum(bit_errors)

pbe[idx] = num / len(bits)

x = np.sqrt(snr)

q = norm.sf(x)

pse = 2*q - q**2

ber[idx] = pse/2

plt.figure(figsize=(8,5))

plt.semilogy(snr_db, ber, '-', label="Theoretical BER")

pbe = np.maximum(pbe, 1e-6)

plt.semilogy(snr_db, pbe, '*', label="Simulated BER")

plt.xlabel("SNR (dB)")

plt.ylabel("Probability of Error")

plt.title("BER vs SNR")

plt.grid(True, which='both', linestyle='--', alpha=0.6)

plt.legend()

plt.show()

n = 10

spers = 4

bits = np.random.randint(0, 2, size=n)

symbols = get_symbols(bits)

pulses = np.repeat(symbols, spers)

# show_pulses(pulses)

h = fix_h(spers, n)

snr_db = np.arange(-5, 14, 2.5)

# question_2(h, spers, n)

# question_3(pulses, h, spers, n)

# question_4(pulses, h, spers, n, bits)

# question_5(pulses, h, spers, n)

# question_6(h, spers)

# question_7(pulses, h, spers, n)

# question_8(symbols, pulses, h, spers, n, snr_db)

# question_9(bits, pulses, h, spers, n, snr_db)